Learning the process:

Step 1: sanitize fermenter clean out gear (sodium percarbonate - no rinse cleaner and sanitiser

Mashtun -used for beer brewing and whiskey brewing

Converts starches in the grain to sugars, allows to filter the grain in the liquid

Add the amount of water required - 17L boil to 75

Mashtun want the grain at 65 degrees ,

After about an hour pour woort into fermenter bucket. Cool woort down to 20 degrees (leave over night to cool own its own) we want to change this and cool it down faster. This stops bacteria from being introduced which is the major failure point. Less exposure to risk if you can decrease the time between start and finish.

Then add 7L at 100 degrees.

Sparge, rinse to get the sugar out of the grain.

Few methods - this is a batch sparge

Aim for a fly sparge for our project. Sprinker/mist over the grain, continuous flow to extract sugars. A lot more efficient.

Aiming for specific gravity 1.06 - 1.07L, EBC 5 (colour of liquid that comes out of the grain)

23L Brew, 31L total (8L lost to grain absorption)

17L Mash @ 65 degrees C for 60min

7L = boiling sparge x 2 (10minutes between)